RST® MINI MOLA Device connector M20, IP66/68(3 m; 2 h)/69, male, 3-pole, ~50 / -120 V, 16 A, signal brown

System RST® MINI MOLA - Device connector according to IEC 61535 for permanent installation, self-locking, for simplified or automated internal wiring of luminaires or similar devices, due to the modular design the housing and internal ballast can be wired independently of each other and can be brought together later, male, 3-pole, turned contacts with push-in spring, fastening screw from inside, mechanical coding signal brown, color coding signal brown.

Rated voltage: ~50 / -120 V

Rated current: 16 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Diameter/thread: M20

Wall thicknesses: up to 4 mm

Rated cross-section connecting-channel A: 1.0 – 2.5 mm² (with ferrules up to 1.5 mm²)

Rated cross-section connecting-channel B (solid): 0.5 – 1.0 mm²

Rated cross-section connecting-channel B (fine stranded): 0.75 – 1.0 mm²

Rated cross-section connecting-channel B (ferrules): 0.5 mm²

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 2

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST16I3 S D03 LMBR

Part No.: 46.032.M050.4