gesis® WIV Distribution Box 20 kVA with RST® POWER, gesis® CLASSIC 12/1/2/0/2

System gesis® WIV – Pre-wired system distributor with pluggable infeed via an RST50 or 32 A CEKON three-phase power supply for a maximum connected load of 20 kVA. The outputs are provided via the GST18® installation coupler system approved in accordance with IEC 61535. Each circuit is individually fused, the 16 A outlets are FI-protected. Several connections can be switched on or off via a common main switch. The distribution system consists of switchable and non-switchable outlets. If required, the non-switchable outlets can be used to operate e.g. emergency lighting or cooling systems. They are also suitable for extending existing installations and are easy to put into operation. An adapter cable 3 m, 32 A CEKON to RST50 is included in delivery.

Connected load: 20 kVA

Maximum fuse: 32 A

Connection type infeed: pluggable RST50

Pluggable system outputs: GST18® and safety sockets

Total output power: 20 kVA

Switchable 1-phase outputs: 12

Switchable 3-phase outputs: 1

Non-switchable 1-phase outputs: 2

Safety socket: 2

Residual current circuit breaker, 4-pole, 40 A, 30 mA: 1

Miniature circuit breaker, 1-pole, B, 16 A: 14

Miniature circuit breaker, 3-pole, B, 16 A: 1

Main switch 63 A: 1

Housing material: polycarbonate

Housing color: electric grey

Ambient conditions: for dry rooms only

Protection class without mating connectors: IP20

Protection class all mating connectors connected: IP40

Mounting method: wall mounting

Depth: 155 mm

Width: 315 mm

Height: 630 mm

Brand: Wieland Electric GmbH

Type: WIV 20 P GC 12-1-2-0-2

Part No.: 93.053.6009.0