gesis® Snap-In, female, 3-pole, for panel mounting, 50 V, 3 A, black

System gesis® NV - Connectors for bus/signal cables (e.g. LON), female version, 3-pole, with spring clamp connection, snapable in housing, with integrated locking, mechanical coding code 3, color coding black.

Rated voltage: 50 V

Rated current: 3 A

Type of protection: IP20

Terminal cross-section, solid: 0.25 - 0.75 mm²

Terminal cross-section, stranded: 0.25 - 0.75 mm²

Housing material: Polyamide

Connections per pole: 1

Marking: 1+, 2-

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: BST14i3 F B1 R SW

Part No.: 93.431.2353.1