gesis® connector, female, 5-pole, 250 V, 20 A, pastel blue

System gesis® CLASSIC - Installation connector according to IEC 61535 for permanent connection, for dimming applications, female version, 5-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 2, color coding pastel blue.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7 - 13.5 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: L, N, PE, D2, D1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i5 S B1 Z R1 PB02

Part No.: 92.953.4453.0