gesis® connector, female, 4-pole, 250/400 V, 20 A, turquoise blue

System gesis® CLASSIC - Installation connector according to IEC 61535 for permanent connection, female version, 4-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical coding code 3, color coding turquoise blue.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: IP40

Terminal cross-section, solid: 0.75 - 2.5 mm²

Terminal cross-section, stranded: 0.75 - 2.5 mm²

Cable diameter: 7.1 - 12.0 mm (H05VV, NYM)

Housing material: Polyamide

Connections per pole: 1

Marking: 4, 3, 2/N, 1

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: GST18i4 S B1 Z R1 TB03

Part No.: 92.943.3353.0