RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 BL

Part No.: 4L.042.4043.9