RST® circular connector, IP66/68(3 m; 2 h)/69, female, angled, 6-pole, 250/400 V, 20 A, with contact seal, turquoise

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m; 2 h)/69, female, 90° angled, 6-pole, turned contacts with screw connection, with contact seal, compact housing with strain relief, field assembly, mechanical/color coding turquoise.

Rated voltage: 250/400 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m; 2 h)/69

Terminal cross section: 0.2 - 2.5 mm²

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2, Ls

Halogen free: yes

Brand: Wieland Electric GmbH

Type: RST20I6S B1 YR1 TB04

Part No.: 9L.063.4053.6