RST® multi distribution unit, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 7 outputs, 250 V, 20 A, signal brown

System RST® CLASSIC - Distribution unit for circular connector IP66/68(3 m; 2 h)/69 according to IEC 61535, distribution unit for optional mounting of up to eight M25 device connectors IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 7 outputs, ready wired, halogen-free wires with 2.5 mm².

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 7

Halogen-free: yes

Dimension: 104x162x96 mm

Brand: Wieland Electric GmbH

Type: RST20I2B 7P1 F VG BR01

Part No.: 99.946.0000.7