RST® circular connector, IP66/68(3 m; 2 h)/69, female, angled, 3-pole, 250 V, 20 A, light grey

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m; 2 h)/69, female, 90° angled, 3-pole, turned contacts with crimp connection, compact housing with strain relief, field assembly, mechanical coding light grey/black, color coding light grey.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m; 2 h)/69

Terminal cross-section, fine-stranded: 0.75 - 4.0 mm²

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST20I3C B1 YR2 V GL

Part No.: 96.133.0153.0