RST® device connector M20, IP66/68(3 m; 2 h)/69, female, angled, 3-pole, low voltage, signal brown

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 3-pole, 90° angled, turned contacts with screw connection fastening screw from inside, mechanical/color coding signal brown.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m; 2 h)/69

Thread: M20x1.5

Terminal cross-section: 0.75 - 6.0 mm²

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST20I3S B1 M10L BR01

Part No.: 96.033.6051.4