RST® device connector M16, IP66/68(3 m; 2 h)/69, female, 3-pole, 250 V, 20 A, black

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 3-pole, turned contacts with screw connection fastening screw from inside, mechanical coding light grey/black, color coding black.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m; 2 h)/69

Thread: M16x1.5

Terminal cross-section: 0.75 - 6.0 mm²

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST20I3S B1 M03 SW

Part No.: 96.031.6153.1