RST® circular connector, IP66/68(3 m; 2 h)/69, male, 2-pole, low voltage, pebble grey

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m; 2 h)/69, male with locking, 2-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m; 2 h)/69

Terminal cross-section, solid: 0.5 - 2.5 mm²

Terminal cross-section, fine-stranded: 0.5 - 1.5 mm²

Terminal cross-section, fine-stranded: 0.75 - 1.5 mm²

For AS-i profile cable

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: -, +

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST20I2F S23ZRASS L KG06

Part No.: 96.022.0950.8