RST® circular connector, IP66/68(3 m; 2 h)/69, female, 2-pole, low voltage, pebble grey

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m; 2 h)/69, female, 2-pole, turned contacts with screw connection, compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250 V

Rated current: 20 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m; 2 h)/69

Terminal cross-section: 0.75 - 6.0 mm²

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: +, -

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST20I2S B13ZR1S L KG06

Part No.: 96.021.4150.8